Section 176.340  Reporting and Cleanup of Spills and Overfills

a)	Owners or operators of USTs shall contain and immediately clean up a spill or overfill, immediately report either release to the 911 call center and then to IEMA, begin initial response and initial abatement in accordance with Sections 176.310, 176.320 and 176.350, and begin corrective action pursuant to 35 Ill. Adm. Code 734, in the following situations:

1)	Spill or overfill of petroleum that results in a release to the environment that exceeds 25 gallons or that causes a sheen on a nearby body of water; or

2)	Spill or overfill of a hazardous substance that results in a release to the environment that equals or exceeds the reportable quantity (see 41 Ill. Adm. Code 174.100). Under Section 176.320, this kind of release shall also be immediately reported to the Local Emergency Planning Committee and to the National Response Center.

b)	Owners or operators of USTs shall contain and immediately clean up a spill or overfill of petroleum that is 25 gallons or less and a spill or overfill of a hazardous substance that is less than the reportable quantity. In doing so, the owner or operator shall comply with procedures specified in Section 176.350. If cleanup cannot be accomplished within 24 hours, owners or operators shall immediately notify IEMA and the local authority having jurisdiction of the release.

(Source:  Amended at 42 Ill. Reg. 10621, effective October 13, 2018)