Section 14.APPENDIX H   Heliport/Vertiport Standards, Facility Requirements and Restrictions on Use

Section 14.TABLE A   Heliport Standards

| Standard | Heliport/ Helistop Vertiport/ Vertistop Public-Use | Heliport/ Helistop Vertiport/ Vertistop Private-Use | Heliport/ Helistop Vertiport/ Vertistop Restricted Landing Area | Heliport/ Helistop Vertiport/ Vertistop Hospital |
| --- | --- | --- | --- | --- |
| Minimum TLOF (Helipad) Size* | 1 Rotor Diameter | 1 Rotor Diameter | 1 Rotor Diameter (Recommended) | 1 Rotor Diameter (40’ Minimum) |
| Minimum FATO (Clear Area) Size | 2 x Rotor Diameter (100’ Minimum) | 2 x Rotor Diameter (100’ Minimum) | 2 x Rotor Diameter (100’ Minimum) | 2 x Rotor Diameter (100’ Minimum) |
| Approach/ Departure Path Requirements | 2 – 90º Apart Minimum (with 2:1 side transition) 8:1 Min. Slope | 2 – 90º Apart Minimum (with 2:1 side transition) 8:1 Min. Slope | 2 – 90º Apart Minimum 1st Approach − 8:1 Min. Slope 2nd Approach − may be 5:1 Min. Slope if necessary | 2 − 90º Apart Minimum 1st Approach − 8:1 Min. Slope 2nd Approach − may be 5:1 Min. Slope if necessary |

*NOTE:  A 6” REINFORCED CONCRETE TLOF IS RECOMMENDED.  If a concrete TLOF is not used, the FATO edges and center must be marked.